IP Address Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| CSE Router | Gig0/0 | 14.52.4.66 | 255.255.255.248 | N/A |
|  | Gig0/2 | 14.52.0.1 | 255.255.255.0 | N/A |
|  | Se0/0/0 | 14.52.4.73 | 255.255.255.252 | N/A |
|  | Se0/1/0 | 14.52.4.81 | 255.255.255.252 | N/A |
|  | Se0/0/1 | 14.52.4.77 | 255.255.255.252 | N/A |
| MNS Router | Gig0/1 | 14.52.2.1 | 255.255.255.0 | N/A |
|  | Se0/0/0 | 14.52.4.74 | 255.255.255.252 | N/A |
| BBS Router | Gig0/0 | 14.52.4.67 | 255.255.255.248 | N/A |
|  | Gig0/2 | 14.52.1.1 | 255.255.255.0 | N/A |
| EEE Router | Gig0/0 | 14.52.4.68 | 255.255.255.248 | N/A |
|  | Gig0/2 | 14.52.3.129 | 255.255.255.128 | N/A |
| BIL Router | Gig0/0 | 14.52.3.1 | 255.255.255.128 | N/A |
|  | Se0/0/0 | 14.52.4.85 | 255.255.255.252 | N/A |
|  | Se0/0/1 | 14.52.4.82 | 255.255.255.252 | N/A |
| LAW Router | Gig0/0 | 14.52.4.1 | 255.255.255.192 | N/A |
|  | Se0/0/0 | 14.52.4.86 | 255.255.255.252 | N/A |

|  | Se0/0/1 | 14.52.4.78 | 255.255.255.252 | N/A |
| --- | --- | --- | --- | --- |
| PC0 | Fa0 | 14.52.3.136 | 255.255.255.128 | 14.52.3.129 |
| PC1 | Fa0 | 14.52.3.134 | 255.255.255.128 | 14.52.3.129 |
| PC2 | Fa0 | 14.52.1.2 | 255.255.255.0 | 14.52.1.1 |
| PC3 | Fa0 | 14.52.1.3 | 255.255.255.0 | 14.52.1.1 |
| PC4 | Fa0 | 14.52.0.5 | 255.255.255.0 | 14.52.0.1 |
| PC5 | Fa0 | 14.52.0.6 | 255.255.255.0 | 14.52.0.1 |
| PC6 | Fa0 | DHCP | 255.255.255.128 | 14.52.3.1 |
| PC7 | Fa0 | DHCP | 255.255.255.128 | 14.52.3.1 |
| PC8 | Fa0 | DHCP | 255.255.255.192 | 14.52.4.1 |
| PC9 | Fa0 | DHCP | 255.255.255.192 | 14.52.4.1 |
| PC10 | Fa0 | DHCP | 255.255.255.0 | 14.52.2.1 |
| PC11 | Fa0 | DHCP | 255.255.255.0 | 14.52.2.1 |
| Printer 0 | 0 | 14.52.3.132 | 255.255.255.128 | 14.52.3.129 |
| Printer 1 | 0 | 14.52.1.4 | 255.255.255.0 | 14.52.1.1 |
| Printer 2 | 0 | 14.52.0.7 | 255.255.255.0 | 14.52.0.1 |
| Printer 3 | 0 | 14.52.3.4 | 255.255.255.128 | 14.52.3.1 |
| Printer 4 | 0 | 14.52.4.3 | 255.255.255.192 | 14.52.4.1 |
| Printer 5 | 0 | 14.52.2.13 | 255.255.255.0 | 14.52.2.1 |
| Email Server 0 | Fa0 | 14.52.0.2 | 255.255.255.0 | 14.52.0.1 |
| Email Server 1 | Fa0 | 14.52.1.5 | 255.255.255.0 | 14.52.1.1 |
| Email Server 2 | Fa0 | 14.52.3.130 | 255.255.255.128 | 14.52.3.129 |
| Email Server 3 | Fa0 | 14.52.3.3 | 255.255.255.128 | 14.52.3.1 |
| Email Server 4 | Fa0 | 14.52.4.2 | 255.255.255.192 | 14.52.4.1 |
| Email Server 5 | Fa0 | 14.52.2.2 | 255.255.255.0 | 14.52.2.1 |
| DHCP Server 1 | Fa0 | 14.52.4.4 | 255.255.255.128 | 14.52.4.1 |
| DHCP Server 2 | Fa0 | 14.52.3.2 | 255.255.255.128 | 14.52.3.1 |
| CSE Web Server | Fa0 | 14.52.0.3 | 255.255.255.0 | 14.52.0.1 |
| MNS Web Server | Fa0 | 14.52.2.3 | 255.255.255.0 | 14.52.2.1 |
| DNS Server | Fa0 | 14.52.0.4 | 255.255.255.0 | 14.52.0.1 |

# Configuration Commands

Router Configuration

cse router

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/2

Router(config-if)# ip address 14.52.0.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/0/0

Router(config-if)# ip address 14.52.4.73 255.255.255.252

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/0/1

Router(config-if)# ip address 14.52.4.77 255.255.255.252

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/1/0

Router(config-if)# ip address 14.52.4.81 255.255.255.252

Router(config-if)# no shutdown

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 14.52.4.66,255.255.255.248

Router(config-if)# no shutdown

bil router

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 14.52.3.1 255.255.255.128

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/0/1

Router(config-if)# ip address 14.52.4.82 255.255.255.252

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/0/0

Router(config-if)# ip address 14.52.4.85 255.255.255.252

Router(config-if)# no shutdown

law router

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 14.52.4.1 255.255.255.192

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/0/1

Router(config-if)# ip address 14.52.4.78 255.255.255.252

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/0/0

Router(config-if)# ip address 14.52.4.86 255.255.255.252

Router(config-if)# no shutdown

mns router

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/1

Router(config-if)# ip address 14.52.2.1 255.255.255.0

Router(config-if)# ip helper-address 14.52.0.4

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface Serial0/0/0

Router(config-if)# ip address 14.52.4.74 255.255.255.252

Router(config-if)# no shutdown

eee router

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/2

Router(config-if)# ip address 14.52.3.129 255.255.255.128

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 14.52.4.68,255.255.255.248

Router(config-if)# no shutdown

bbs router

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/2

Router(config-if)# ip address 14.52.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 14.52.4.67,255.255.255.248

Router(config-if)# no shutdown

central switch

Router> enable

Router# configure terminal

Switch(config)# interface vlan 1

Switch(config-if)# ip address 14.52.4.65 255.255.255.248

Switch(config-if)# no shutdown

DHCP Configuration

en conf t

ip dhcp excluded-address 14.52.0.1 14.52.0.20         ip dhcp excluded-address 14.52.1.1 14.52.1.10         ip dhcp excluded-address 14.52.2.1 14.52.2.10

ip dhcp excluded-address 14.52.3.129 14.52.3.140

ip dhcp pool CSE_POOL network 14.52.0.0 255.255.255.0 default-router 14.52.0.1 dns-server 14.52.0.4 exit

ip dhcp pool BBS_POOL network 14.52.1.0 255.255.255.0 default-router 14.52.1.1 dns-server 14.52.0.4 exit

ip dhcp pool MNS_POOL network 14.52.2.0 255.255.255.0 default-router 14.52.2.1 dns-server 14.52.0.4 exit

ip dhcp pool EEE_POOL network 14.52.3.128 255.255.255.128 default-router 14.52.3.129 dns-server 14.52.0.4 exit

en conf t int g0/1   ip helper-address 14.52.4.73 exit

en

conf t int g0/2   ip helper-address 14.52.4.66 no shutdown exit

en conf t int g0/2   ip helper-address 14.52.4.67 exit

en conf t

ip dhcp excluded-address 14.52.4.1 14.52.4.10   ip dhcp pool LAW_POOL network 14.52.4.0 255.255.255.192 default-router 14.52.4.1 dns-server 14.52.0.4  exit

en conf t ip dhcp excluded-address 14.52.3.1 14.52.3.10

ip dhcp pool BIL_POOL network 14.52.3.0 255.255.255.128 default-router 14.52.3.1 dns-server 14.52.0.4 exit

Routing Configuration

en conf t router rip version 2 no auto-summary network 14.52.0.0 network 14.52.4.64 exit

en conf t router rip version 2 no auto-summary network 14.52.3.128 network 14.52.4.64 exit

en conf t router rip version 2 no auto-summary network 14.52.1.0 network 14.52.4.64 exit

en conf t

ip route 0.0.0.0 0.0.0.0 14.52.4.73

en conf t ip route 14.52.2.0 255.255.255.0 14.52.4.74    ip route 14.52.4.0 255.255.255.192 14.52.4.78  ip route 14.52.3.0 255.255.255.128 14.52.4.82 exit

en conf t ip route 0.0.0.0 0.0.0.0 14.52.4.77 ip route 0.0.0.0 0.0.0.0 14.52.4.85 10 exit

en conf t ip route 0.0.0.0 0.0.0.0 14.52.4.81 ip route 0.0.0.0 0.0.0.0 14.52.4.86 10

exit

en conf t ip route 14.52.2.0 255.255.255.0 14.52.4.66 exit

en conf t ip route 14.52.2.0 255.255.255.0 14.52.4.66 exit

en conf t router rip version 2 redistribute static exit